Secondary Storage Devices

Limitations of Primary Storage

- Limited Capacity: It is often necessary to store many millions of bytes of data in a computer. Unfortunately, the storage capacity of the primary storage of computers is not sufficient to store the large volume of data which needs to be handled by most data processing centres.

- Volatile: Primary storage is volatile. However, computer systems need to store data on a permanent basis for long periods of time.

Need for Secondary Storage

As a result of the limitations of primary storage, additional memory called secondary storage is used with most computer systems.

This section of the computer’s memory is non-volatile, has lower cost per bit stored, but it generally has an operating speed far slower than that of primary storage.

Secondary storage is used to store large volumes of data on a permanent basis, which can be partially transferred to primary storage as and when required for processing.

Sequential and Direct-Access Devices

Several devices are used for secondary storage, but the one selected for a particular application depends upon how the stored information needs to be accessed.

There are two methods used for accessing information:

- Sequential or serial access

- Direct or random access

Sequential access devices

Information on a sequential access device can only be retrieved in the same sequence in which it is stored.

Analogy: audio cassette

Example: magnetic tape

Direct access devices

Such devices permit access to individual information in a more direct manner.

They are also called random-access devices because the information is literally available at random i.e. in any order.

Analogy: audio cd’s

Example: magnetic disks (floppy disk, hard disk), optical disks (CD-ROM)

Magnetic Tape

Magnetic tape is the most popular storage medium for large data which are sequentially accessed and processed.

The magnetic tape medium is a plastic ribbon, usually ½ or ¼ inch wide and 50 – 2400 feet long.

It is coated with a magnetizable recording material e.g. iron oxide. Data are recorded on the tape in the form of tiny invisible magnetized and non-magnetized spots (representing 1s and 0s) on the coated surface of the tape.

The tape ribbon is itself stored in reels or a small cartridge or cassette.

Like audio or videotape, a magnetic tape can be erased and used indefinitely. Old data on the tape are automatically erased as new data are recorded in the same area. However, the information stored can be read many times, without affecting the stored data.

Advantages of Magnetic Tapes

- Their storage capacity is virtually unlimited, because as many tapes as required can be used for storing very large data sets.

- With the low cost of tape reels and cartridges and high data recording densities, the cost per bit of storage is very low for magnetic tapes.

- Tape reels are easy to handle and store since they are compact and light in weight.

- Due to their compact size and light weight, they are easily portable. They are often used for transferring data and programs from one computer to another.

Limitations of Magnetic Tapes

- They are not suitable for storage of data which frequently requires to be accessed randomly due to their sequential access nature.

- They must be stored in a dust-free environment, because specks of dust can cause tape-reading errors.

- They must also be stored in an environment with properly controlled temperature and humidity levels otherwise the tape ribbon might get twisted due to warping, resulting in loss of stored data.

Uses of Magnetic Tapes

- Backing up of data stored on an online storage device, such as a disk, so that if by accident, the data on the disk is corrupted or lost, it can be retrieved from the backup tape.

- Archiving data which are not frequently used.

- Transferring data and programs from one computer to another.

- Distribution of software by vendors. Originally sold software or software updates are often distributed by vendors on magnetic tapes.

Magnetic Disk

Magnetic disks are the most popular on-line secondary storage devices because of their random access capability.

A magnetic disk is a thin, circular plate made of metal or plastic which is usually coated on both sides with a magnetizable recording material such as iron oxide.

Data is recorded on the disk in the form of tiny invisible magnetized and non-magnetized spots on the coated surfaces of the disk.

The disk itself is stored in a specially designed protective envelope or cartridge, or several of them may be stacked together in a sealed, contamination-free container.

Magnetic disks, just like magnetic tapes, can be erased and reused indefinitely. Information stored can be read several times, without affecting the stored data.

Access Time

Disk access time is the interval between the time a computer makes a request for transfer of data from a disk system to the primary storage and the time this operation is completed.

To access information stored on a disk, the disk address of the desired data has to be specified.

Types of Magnetic Disks

All magnetic disk may be broadly classified into two types:

- Floppy disks

- Hard disks

Floppy Disks

A floppy disk is a round, flat piece of flexible plastic, coated with magnetic oxide.

It is encased in a square plastic or vinyl jacket cover. The jacket gives handling protection to the disk surface. Moreover, it has a special liner, which provides a wiping action to remove dust particles that are harmful for the disk surface and read/write head.

Floppy disks are so called because they are made of flexible plastic plates which can bend.

They are also known as floppies or diskettes.

Floppy Disk Drive

A floppy-disk drive is a device used to read/write data from/to floppy disks. The drive has a spindle which rotates the disk, and read/write heads which can move in and out to position themselves on any track of the disk surface.

A floppy-disk drive allows the loading and unloading of magnetic disks as and when they are needed for reading/writing data to them.

The read/write heads of a floppy-disk drive make direct contact with the disk surface during the process of reading or writing. Hence, floppy disks get worn out with constant use.

Hard Disks

Hard disks are the primary online secondary storage device for most computer systems today.

Hard disks are made of rigid metal (frequently aluminium).

Types of hard disks: zip disks, disk packs, winchester disks

Advantages of Magnetic Disks

- Unlike magnetic tapes, magnetic disks support direct access of data. Hence they are more suitable for a wider range of applications.

- Due to the property of random access, magnetic disks are often used simultaneously by multiple users as a shared device.

- Magnetic disks are suitable for both on-line and off-line storage of data. E.g. floppy disks are used as offline secondary storage devices whereas hard disks are used as online secondary storage devices.

- Due to their low cost and high data recording densities, the cost per bit of storage is low for magnetic disks. An additional cost benefit is that magnetic disks can be erased and used many times.

- Floppy disks and zip disks are compact and light in weight. Hence they are easy to handle and store. Very large amount of data can be stored in a small storage space.

- Any information desired from a disk storage can be accessed in a few milliseconds because it is a direct access storage device. This is not possible in the case of a tape storage,

- Data transfer rate for a magnetic disk system is normally higher than a tape system.

- Magnetic disks are less vulnerable to data corruption due to careless handling or unfavourable temperature and humidity conditions than magnetic tapes.

Limitations of Magnetic Disks

- Magnetic disks must be stored in a dust-free environment.

- For some type of magnetic disks, a disk crash or drive failure often results in the loss of entire data stored on it. It is not easy to recover the lost data.

- On a cost-per-bit basis, the cost of magnetic disks is low, but the cost of magnetic tapes is even lower.

Uses of Magnetic Disks

- For applications that are based on random data processing.

- Distribution of software by vendors.

- Transferring of data and programs from one computer to another.

- As a backup device for off-line storage of data.

- Archiving of data which are not used frequently.

Optical Disk

An optical disk storage medium consists of a rotating disk, which is coated with a thin metal or some other material that is highly reflective.

Laser beam technology is used for recording/reading of data on the disk.

Due to the use of laser beam technology, optical disks are also known as laser disks or optical laser disks.

Types of Optical Disks

All optical disks are round platters. They come in different sizes and capacities.

Optical disks can store much more data than most portable magnetic media.

There are three basic types of optical disks:

- CD-ROM

- WORM

- Erasable Optical Disks

CD-ROM

CD-ROM stands for Compact Disk Read Only Memory.

Like audio CD’s, CD-ROMs come with data already encoded onto them.

The data is permanent and can be read any number of times, but CD-ROMs cannot be modified.

WORM

Stands for write-one, read-many. With a WORM disk drive, the disk can be read and re-read, but once it is recorded, it cannot be changed,

After that, the WORM disk behaves just like a CD-ROM.

The WORM drive is a high-capacity storage device and is best for storing archives and other large amounts of unchanging information.

Erasable-Optical disks

These disks can be erased and loaded with new data, just like magnetic disks.

These disks are written and read using a laser beam, but the storage process is really magnetic rather than optical.

They are also known as magneto-optical disks.

Advantages of Optical Disks

- The cost per bit of storage for optical disks is ver low, because of their low cost and enormous storage density.

- The use of a single spiral track makes optical disks an ideal storage medium for reading large blocks of sequential data, such as music.

- Optical disk drives do not have any mechanical read/write heads to rub against or crash into the disk surface. This makes optical disks a more reliable storage medium than magnetic tapes or magnetic disks.

- Optical disks have a data storage life in excess of 30 years. This makes them a better storage medium for data archiving as compared to magnetic tapes or magnetic disks.

- Since data once stored on an optical disk becomes permanent, the danger of stored data getting inadvertently erase/overwritten is not there with optical disks.

- Due to their compact size and light weight, optical disks are easy to handle, store and move from one place to another.

Limitations of Optical Disks

- The data access speed for optical disks is slower than magnetic disks.

- Optical disks require a more complicated drive mechanism than magnetic disks.

Uses of Optical Disks

- For distributing large amounts of data at a low cost.

- For distribution of electronic version of conference proceedings, journals, magazines, books, product catalogues, etc.

- For distribution of new or upgraded versions of software products by software vendors.

- For storage and distribution of a wide variety of multimedia applications, such as video games.

- For archiving of data which are not used frequently.